SINGLE/MULTI STAGE AMPLIFIERS 설계

What is Amplifier ?

It is an electronic device that increases the power of a signal. It does this by taking energy from a power supply and controlling the output to match the input signal shape but with a larger amplitude.

Types of Bipolar Transistors

- The Bipolar Junction Transistor (BJT) is a three layers device constructed form two semiconductor diode junctions joined together, one forward biased and one reverse biased.

- There are two main types of bipolar junction transistors, the NPN and the PNP transistor.

- Transistors are “Current Operated Devices” where a much smaller Base current causes a larger Emitter to Collector current, which themselves are nearly equal, to flow.

- The arrow in a transistor symbol represents conventional current flow.

- Requires a Biasing voltage for AC amplifier operation.

- The Base-Emitter junction is always forward biased whereas the Collector-Base junction is always reverse biased.

- The standard equation for currents flowing in a transistor is given as:  IE = IB + IC

- The Collector or output characteristics curves can be used to find either Ib, Ic or β to which a load line can be constructed to determine a suitable operating point, Q with variations in base current determining the operating range.

- A transistor can also be used as an electronic switch between its saturation and cut-off regions to control devices such as lamps, motors and solenoids etc..

- The NPN transistor requires the Base to be more positive than the Emitter while the PNP type requires that the Emitter is more positive than the Base.

Bias Conditions for BJT

Types of Amplifiers

- The most common transistor connection is the Common Emitter (CE) configuration but Common Base (CB) and Common Collector (CC) are also available.

Common Emitter Amplifiers

Common Base Amplifiers

Common Collector Amplifiers (Emitter Follower)

Voltage Divider Bias Circuit for BJT Amplifiers

The voltage divider is formed using external resistors R1 and R2. The voltage across R1 forward biases the emitter junction. By proper selection of resistors R1 and R2, the operating point of the transistor can be made independent of β. In this circuit, the voltage divider holds the base voltage fixed independent of base current provided the divider current is large compared to the base current. However, even with a fixed base voltage, collector current varies with temperature (for example) so an emitter resistor is added to stabilize the Q-point, similar to the above circuits with emitter resistor. The voltage divider configuration achieves the correct voltages by the use of resistors in certain patterns. By manipulating the resistors in certain ways you can achieve more stable current levels without having β value affect it too much.

We first analyze the bias circuit under DC conditions. The Thevenin equivalent voltage and resistance from base to ground are

VBB= R1R1+R2∙VCC

RBB=R1∥R2=R1R2R1+R2

For R2

VBB= RBBR2VCC     →    R2= VCCRBBVBB

For R1

R1+ R2VBB= R1VCC

VCC- VBBR1= R2VBB

R1= R2VBBVCC- VBB= VCCRBBVBB  VBBVCC- VBB=      RBB1- VBBVCC

Writing KVL equations around base loop, we obtain

VBB= IBQRBB+VBE+IEQRE       ←    IBQ=ICQβ ,   IEQ≈ICQ

VBB≈ ICQRBBβ+VBE+ICQRE

and at the quiescent point,

ICQ= VBB- VBERBBβ+ RE

We neglect VBE since we are dealing only time-varying components. The current restriction then becomes

RBBβ ≪ RE

Generally, RBB is to be set

RBB ≈ 110 β RE

This prevents variation in β from significantly affecting the dc operating point of the stage.  The above equations are used in the design process.

Merits:

- Unlike above circuits, only one dc supply is necessary.

- Operating point is almost independent of β variation.

- Operating point stabilized against shift in temperature.

Demerits:

- As β-value is fixed for a given transistor, this relation can be satisfied either by keeping RE fairly large, or making R1||R2 very low.

  - If RE is of large value, high VCC is necessary. This increases cost as well as precautions necessary while handling.

  - If R1 || R2 is low, either R1 is low, or R2 is low, or both are low. A low R2 raises VB closer to VC, reducing the available swing in collector voltage, and limiting how large RC can be made without driving the transistor out of active mode. A low R1 lowers Vbe, reducing the allowed collector current. Lowering both resistor values draws more current from the power supply and lowers the input resistance of the amplifier as seen from the base.

- The standard voltage divider circuit discussed above faces a drawback – AC feedback caused by resistor RE reduces the gain. This can be avoided by placing a capacitor (CE) in parallel with RE, as shown in circuit diagram. The result is that the DC operating point is well controlled, while the AC-gain is much higher (approaching β), rather than the much lower (but predictable) value of R c / R e {\displaystyle R_{\text{c}}/R_{\text{e}}} without the capacitor.

Usage:

The circuit's stability and merits as above make it widely used for linear circuits.

How to find Q-point for maximum output swing using AC and DC Load Lines

DC circuit

AC circuit

Rdc= RC+ RE

Rac= RC∥RL+RE

Q-point of transistor for maximum output swing

Equation of DC Load Line

y= -1Rdcx-VCC

Equation of AC Load Line

y-ICQ = -1Racx-VCEQ

y= -1Racx+ VCEQRac+ ICQ

If   x=0,  y= VCEQRac+ ICQ= I`C

If   y=0,  x= VCEQ+ ICQRac= V`CC

Bias voltage condition for maximum output swing

Maximum collector current on AC load line

I`C = VCEQRac+ ICQ=2 ICQ

ICQ= VCEQRac  → VCEQ= RacICQ

Bias current condition for maximum output swing

For VCEQ  use  AC and DC load lines

-1RacVCEQ+ VCEQRac+ ICQ = -1RdcVCEQ-VCC     ←     ICQ= VCEQRac

VCEQ1Rac+ 1Rdc= VCCRdc       →       VCEQ1+ RdcRac= VCC

VCEQ= VCC1+ RdcRac

ICQ= VCEQRac= VCCRac+ Rdc

Analysis of CE Amplifier

Input resistance ;

Rin= RBB∥rπ+1+βRE≈RBBrπ+βRERBB+rπ+βRE=RBBrπβ+RERBBβ+rπβ+RE

=RBBre+RERBBβ+re+RE      where  re=VTICQ

Current gain ;

Ai=iLiin             ib≈RBB iinRBB+rπ+βRE        iL≈-βibRCRC+RL

Ai=--βRCRC+RLibRBB+rπ+βRERBBib=-βRBBRCRBB+rπ+βRERC+RL

=RBBRCRBBβ+re+RERC+RL

Voltage gain ;

Av=vovin=iLiinRLRin=-RBBRCRLRBBβ+re+RERC+RL RBBβ+re+RERBBre+RE

=-RC∥RLre+RE

Design Procedure of CE Amplifier

Step 1.  Determine the value of RC using table

|  | Bias Current | Current Gain | vo(p-p) |
| --- | --- | --- | --- |
| RC = 1•RL | Medium | Medium | Medium |
| RC = 0.1•RL | High | Low | High |
| RC = 10•RL | Low | High | Low |

Step 2.  Find Rdc and Rac

Rdc= RC+ RE

Rac= RC∥RL+ RE   for  CE amp. without bypass capacitor

Rac= RC∥RL   for  CE amp. with bypass capacitor

Step 3.  Calculate the quiescent collector current, ICQ, needed to place the Q-point in the center of the AC load line(i.e. Maximum swing)

ICQ=  VCCRac+Rdc

VCEQ= RacICQ

Step 4.  Determine R`E using the voltage gain specification.

Av = RC∥RLR`E   for  CE amp. without bypass capacitor

Step 5.  Find small signal emitter resistance, re, and RE.

re= VTICQ= 26 mVICQ

RE= R`E- re

Step 6.  If there are a current gain or input resistance specification for the design, we shall use it to solve for the value of bias resistance RBB. If not, we use the bias stability condition.

RBB=0.1βRE

Step 7.  Calculate the quiescent collector-emitter voltage, VCEQ.

VCEQ= RacICQ

Step 8.  Calculate the quiescent base voltage, VBB.

VBB=ICQβ RBB+ VBE+ICQRE

Step 9.  Calculate bias resistors R1 and R2.

R2=VCCRBBVBB

R1= RBB1- VBBVCC

Design of Bias Circuit

Example: Select R1 and R2 for maximum output voltage swing in the circuit shown in the below figure.

SOLUTION:  We first determine ICQ for the circuit.

Rac= RC∥RL = 500 Ω

Rdc = RE + RC =1100 Ω

ICQ=VCCRac+Rdc=5500+1100=3.13 mA

For maximum swing,¡

V`CC = 2•VCEQ

VCEQ = ICQ•Rac = (3.13 mA)(500Ω) =1.56 V

V`CC = 2•VCEQ = 3.13 V

From the manufacturer`s specification, the maximum β for the 2N3903 is 200.  RB is set equal to 0.1(β)(RE) for bias stability,

RBB = 0.1(200)(100) = 2 kΩ

VBB=ICQRBBβ+RE+VBE=3.13×10-32000200+100+0.7=1.044 V

Since we know VBB and RBB, we find R1 and R2

:

R1=RBB1-VBBVCC=20001-1.0445=2.53 kΩ

R2=RBBVCCVBB=2000×51.044=9.6 kΩ

The maximum output voltage swing, ignoring the nonlinearities at saturation and cutoff, would then be

Maximum output swing= 2•ICQ•( RC∥RL) = 2(3.13mA)(500Ω)=3.13 V

We check the maximum power dissipated by the transistor to assure that it will not exceed the specifications.

P(transistor)= VCEQ•ICQ = (1.56 V)•(3.13 mA) = 4.87 mW

This is well within the 350 mW maximum given on the specification.

Capacitor-Coupled CE Amplifier Design

Design a CE amplifier with Av=-10, β=200, and RL=1 kΩ.  A pnp transistor is used and maximum symmetrical output swing is required.  Check the value of Av using a computer simulation.

SOLUTION :

We shall choose RC = RL = 1 kΩ

Using the equation for Av to solve for RE,

AV= - RC∥RLR`E        where    R`E=re+RE

When known values are substituted into this equation, we find R`E=50 Ω.

We need to know the value of re in order to find RE. We first find Rac and Rdc.

Rac = RE + (RC∥RL) = 550 Ω

Rdc = RE + RC = 1050 Ω

With Rac and Rdc determined, the design of this circuit now parallels the step-by-step procedure for the design. The first step is to calculate the quiescent current needed to place the Q-point in the center of the ac load line. The equation is

ICQ=  -VCCRac+Rdc= -7.5  mA

The quantity re is found as follows

re=VTICQ= 26 mV7.5 mA=3.47 Ω

Then

RE = 50 – re = 46.5 Ω

If there were a current gain or input resistance specification for this design, we would use it to solve for the value of RBB. Since there is no such specification, we use the expression

RBB = 0.1βRE = 0.1(200)(46.5) = 930 Ω

Then continuing with the design steps,

Ai=-RBBRBBβ+re+RE⋅ RCRC+RL= -8.5

VCEQ=VCC- RC+REICQ= -4.15 V

VBB= ICQRE+RBBβ+ VBE= -1.08 V

R1= RBB1-VBBVCC=1.02 kΩ

R2= RBBVCCVBB=10. 3 kΩ

Rin= RBBre+RERBBβ+re+RE=851 Ω

RO= RC=1 kΩ

The maximum peak-to-peak output swing is given by

vo(p-p)=2ICQRc∥RL=7.5 V

The power delivered into the load is given by

PLoad=12ICQRCRC+RL2RL=7 mW

Analysis of CC(Emitter Follower) Amplifier

Input resistance ;

Rin= RBB∥rπ+1+βRE∥RL≈RBBrπ+βRE∥RLRBB+rπ+βRE∥RL

=RBBrπβ+RE∥RLRBBβ+rπβ+RE∥RL =RBBre+RE∥RLRBBβ+re+RE∥RL

Current gain ;

Ai=iLiin             ib≈RBB iinRBB+rπ+βRE∥RL        iL≈βibRERE+RL

Ai=βRERE+RLibRBB+rπ+βRE∥RLRBBib=βRBBRERBB+rπ+βRE∥RLRE+RL

=RBBRERBBβ+re+RE∥RLRE+RL

Voltage gain ;

Av=vovin=iLiinRLRin

=RBBRERLRBBβ+re+RE∥RLRE+RL RBBβ+re+RE∥RLRBBre+RE∥RL

=RE∥RLre+RE∥RL

Design of Capacitor-Coupled EF Amplifier

Design a single-stage npn Emitter Follower amplifier with β=60, VBE=0.7 V, Rin= 1 kΩ, and VCC= 12 V. Determine the circuit element values for the stage to achieve Ai =10 with a 100 Ω load.

SOLUTION   We must select R1, R2, and RE but we have only two equations. These two equations are specified by the current gain and the placement of the Q-point. Previous example showed that the best choice for a CE amplifier is to make RC=RE. We could derive a similar result for RE and RL in the CC amplifier. We shall therefore begin by constraining RE to be equal to RL. This yield a third equation,

RE=RL= 100 Ω

Now finding the load line slopes,

Rac = RL∥RE = 100 Ω

Rdc = RE = 100 Ω

Since the amplitude of the input is not specified, we choose the quiescent current to place the Q-point in the center of the ac load line for maximum swing.

ICQ=VCCRac+Rdc=80 mA

VCEQ=ICQ∙Rac=4 V

We now find the value of re

re=VTICQ=26 mV80 mA=0.33 Ω

Since re is insignificant compared to RE∥RL, it can be ignored, which is usually the case for EF circuits.

Using the equation for the current gain, we find

Ai=βRERBBRE+RLRBB+RE∥RLβ

Everything in this equation is known except RBB. We solve for RBB with the result

RBB =1500 Ω

VBB is found from the base loop.

VBB=VBE+ICQRBBβ+RE=10.7 V

Continuing with the design as presented earlier, we find

R1= 13.5 kΩ

and

R2= 1.68 kΩ

The voltage gain of the CC amplifier is approximately unity. The input resistance is found

Rin=RB∥[β(RE∥RL)] = 1 kΩ

The output resistance is found

r0=re+RBB∥Rsβ∥RE=9.36 Ω

The maximum peak-to-peak symmetrical output swing is given by

vo(p-p)=1.8ICQRE∥RL=7.2 V

Analysis of CB Amplifier

Input resistance ;

Rin= RE∥rπ+RBB1+β≈RE∥re+RBBβ=REre+RBBβRE+re+RBBβ

Current gain ;

Ai=iLiin             ie≈RE iinRE+re+RBBβ        iL≈ieRCRC+RL

Ai=ieRCRC+RLRE+re+RBBβREie=RCRERE+re+RBBβRC+RL

Voltage gain ;

Av=vovin=iLiinRLRin

=RCRERE+re+RBBβRC+RL RLRE+re+RBBβREre+RBBβ= RC∥RLre+RBBβ

Design of Capacitor-Coupled CB Amplifier

Design a CE amplifier using npn transistor with with β=100, VCC=24 V, RL=2 kΩ, RE=400 Ω, and VBE=0.7 V. Design this amplifier for a voltage gain of 20.  Verify your design using a computer simulation.

SOLUTION :

We shall choose RC = RL = 2 kΩ

Using the equation for Av, we have

AV= RC∥RLre+RBBβ=20      →  re+RBBβ=100020=50 (Ω)

RBB=β50-re

We find Rac and Rdc.

Rac = RE + (RC∥RL) = 1400 Ω

Rdc = RE + RC = 2400 Ω

With Rac and Rdc determined, the next step is to calculate the quiescent current needed to place the Q-point in the center of the ac load line. The equation is

ICQ=  VCCRac+Rdc= 241400+2400≈6.3  mA

re=VTICQ=26 mV6.3 mA≈4.13 Ω

re is much less than RE, so

RBB=β50-re=10050-4.13≈4.59 kΩ

We use bias equation to find the parameters of the input bias circuitry.

VBB= ICQRE+RBBβ+ VBE=6.3m400+4.59k100+ 0.7

VBB≈ 3.52 V

The bias resistors are then given by

R1= RBB1-VBBVCC=5.38 kΩ

R2= RBBVCCVBB=31.3 kΩ

Therefore, we can calculate the input resistance and the current gain

.

Ai=ieRCRC+RLRE+re+RBBβREie=RCRERE+re+RBBβRC+RL≈0.44

Rin= RE∥rπ+RBB1+β≈RE∥re+RBBβ=REre+RBBβRE+re+RBBβ≈44 Ω

The maximum peak-to-peak output swing is given by

vop-p=1.8ICQRc∥RL=11.4 V

Frequency Responses of Amplifier

# Time Constant

All Electrical or Electronic circuits or systems suffer from some form of “time-delay” between its input and output, when a signal or voltage, either continuous, ( DC ) or alternating ( AC ) is firstly applied to it.

This delay is generally known as the time delay or Time Constant of the circuit and it is the time response of the circuit when a step voltage or signal is firstly applied. The resultant time constant of any electronic circuit or system will mainly depend upon the reactive components either capacitive or inductive connected to it and is a measurement of the response time with units of, ( τ )

When an increasing DC voltage is applied to a discharged Capacitor, the capacitor draws a charging current and “charges up”, and when the voltage is reduced, the capacitor discharges in the opposite direction. Because capacitors are able to store electrical energy they act like small batteries and can store or release the energy as required.

The charge on the plates of the capacitor is given as: Q = CV. This charging (storage) and discharging (release) of a capacitors energy is never instant but takes a certain amount of time to occur with the time taken for the capacitor to charge or discharge to within a certain percentage of its maximum supply value being known as its Time Constant ( τ ).

If a resistor is connected in series with the capacitor forming an RC circuit, the capacitor will charge up gradually through the resistor until the voltage across the capacitor reaches that of the supply voltage. The time also called the transient response, required for the capacitor to fully charge is equivalent to about 5 time constants or 5τ .

This transient response time  τ , is measured in terms of  τ  = R x C, in seconds, where R is the value of the resistor in ohms and C is the value of the capacitor in Farads. This then forms the basis of an RC charging circuit were 5τ can also be thought of as “5 x RC”.

## RC Charging Circuit

The figure below shows a capacitor, ( C ) in series with a resistor, ( R ) forming a RC Charging Circuit connected across a DC battery supply ( Vs ) via a mechanical switch. When the switch is closed, the capacitor will gradually charge up through the resistor until the voltage across it reaches the supply voltage of the battery. The manner in which the capacitor charges up is also shown below.

### RC Charging Circuit

Let us assume above, that the capacitor, C is fully “discharged” and the switch (S) is fully open. These are the initial conditions of the circuit, then t = 0, i = 0 and q = 0. When the switch is closed the time begins at t = 0 and current begins to flow into the capacitor via the resistor.

### RC Charging Circuit Curves

The capacitor now starts to charge up as shown, with the rise in the RC charging curve steeper at the beginning because the charging rate is fastest at the start and then tapers off as the capacitor takes on additional charge at a slower rate.

As the capacitor charges up, the potential difference across its plates slowly increases with the actual time taken for the charge on the capacitor to reach 63% of its maximum possible voltage, in our curve 0.63Vs being known as one Time Constant, ( τ ).

This 0.63Vs voltage point is given the abbreviation of 1τ, (one time constant).

The capacitor continues charging up and the voltage difference between Vs and Vc reduces, so to does the circuit current, i. Then at its final condition greater than five time constants  ( 5τ ) when the capacitor is said to be fully charged, t = ∞, i = 0, q = Q = CV. Then at infinity the current diminishes to zero, the capacitor acts like an open circuit condition therefore, the voltage drop is entirely across the capacitor.

So mathematically we can say that the time required for a capacitor to charge up to one time constant, ( 1τ ) is given as:

### RC Time Constant,  τ

τC=C × Req

This RC time constant only specifies a rate of charge where, R is in Ω‘s and C in Farads.

After a time of 5τ the capacitor is now fully charged and the voltage across the capacitor, ( Vc ) is equal to the supply voltage, ( Vs ). As the capacitor is fully charged no more current flows in the circuit. The time period after this 5τ point is known as the Steady State Period.

Note that as the charging curve for a RC charging circuit is exponential, the capacitor in reality never becomes 100% fully charged due to the energy stored in the capacitor. So for all practical purposes, after five time constants a capacitor is considered to be fully charged.

## RC Charging Circuit Example No1

Calculate the RC time constant, τ of the following circuit.

The time constant,  τ is found using the formula  τ = R x C in seconds.

Therefore the time constant τ is given as:   τ = R x C = 47k x 1000uF = 47 secs

Cutoff Frequency vs Time Constant

⒜ 1st order Simple CR Circuit

The transfer function in frequency domain is

HS= VoSVinS= R1SC+R= SS+ 1CR= SS+ 1τC

where τC is time constant

Magnitude response with respect to ω is

Hjw= ωω2+1CR2

If the frequency of input signal approaches to 0, the gain of the transfer function reaches -∞ dB. While the gain of the transfer function reaches 0 dB. When the input signal frequency is 1/RC (rad/sec), the magnitude of H(S) is 12 i.e. -3 dB.

Therefore, the cutoff frequency of the simple C-R circuit is

ωcutoff= 1RC= 1τC  (radsec)

This means that the cutoff frequency is equal to the reciprocal of the time constant of the capacitor in the CR first order circuit.

⒝ 1st order Simple RC Circuit

The transfer function in frequency domain is

H(S)= VoSVinS= 1SC1SC+R= 1RCS+ 1RC= 1RCS+ 1τC

where τC is time constant

Magnitude response with respect to ω is

Hjw= 1RCω2+1RC2

If the frequency of input signal approaches to 0, the gain of the transfer function reaches 0 dB.   While the gain of the transfer function reaches -∞dB when the frequency of the input signal reaches ∞. When the input signal frequency is 1/RC (rad/sec), the magnitude of H(S) is 12 i.e. -3 dB.

Therefore, the cutoff frequency of the simple R-C circuit is

ωcutoff= 1RC= 1τC  (radsec)

This means that the cutoff frequency is equal to the reciprocal of the time constant of the capacitor in the RC first order circuit.

(Example 1) Bandwidth Design

⒞ Miller Effect (Miller Capacitor)

At the input terminal, the relationship between voltage va and current ia is

ia= va- vb1sCF= 1- vbva1va 1sCF= va11- vbva 1sCF= va1s1- AviCF= va1sCM1

CM1= 1- AviCF

At the input terminal, the relationship between voltage vb and current ib is

ib= vb- va1sCF= 1- vavb1vb 1sCF= vb11- vavb 1sCF= vb1s1- 1AviCF= vb1sCM2

CM2= 1- 1AviCF

If Avi is much larger than 1 and is out of phase, CM1 and CM2 are approximately

CM1≈ AviCF

CM2≈ CF

(Example 2)  Miller capacitor

(Example 3) Total Frequency Response of Amplifier

AC circuit of CE amplifier

Mid-Frequency Response of CE Amplifier

Capacitor C1, C2, and CE act as short circuit.

Capacitor CF acts as open circuit.

Voltage gain : Avi= vovi= -gmRC∥RL

Input resistance : Rin= R1∥R2∥rπ

Total Voltage gain : Avs= vovs= -RinRin+RS gmRC∥RL

Current gain : Ai=ioii=voRLvsRS+ Rin=Rin+RSRLAvs  =-RinRL gmRC∥RL

Low-Frequency Response of CE Amplifier

A BJT amplifier is shown in Figure 1. Each circuit has its own cutoff frequency, which is calculated using the relationship shown. Note that the lower cutoff frequencies for the base f1B, collector f1C, and emitter f1E, circuits are calculated. The highest cutoff frequency calculated is the overall value of fC1, for the circuit.

Figure 1  BJT amplifier lower cutoff frequencies

Capacitor CF acts as open circuit. The small signal equivalent circuit in low frequency range is

Lower frequency due to CC1 is

f1B= 12πRS+ RinCC1              Rin= R1∥R2∥rπ

CC1= 12πRS+ Rinf1B

Lower frequency due to CC2 is

f1C= 12πRC+ RLCC2

CC2= 12πRC+ RLf1C

Lower frequency due to CE is

f1E= 12πRoutCE        Rout= rπ+ R1∥R2∥RS1+β= re+ R1∥R2∥RSβ

CE= 12πRoutf1E

Among the lower cutoff frequencies for the base f1B, collector f1C, and emitter f1E, one of them sets to be lower half frequency fC1. Rest of them must set to be smaller than lower half frequency.

For example, if f1E sets to be fC1, then

f1E= fC1      Lower half power frequency

f1B= 110 fC1   and    f1C= 120 fC1

or

f1C= 110 fC1   and    f1B= 120 fC1

High-Frequency Response of CE Amplifier

The high-frequency equivalent of a BJT amplifier is shown in Figure 2 with C1, C2, and CE act as short circuit. Each circuit has its own cutoff frequency, which is calculated using the relationship shown. Note that the upper cutoff frequencies for the base f2B,  and collector f2C,  circuits are calculated, and the lowest result is the overall value of fC2, for the circuit.

Figure 2  BJT amplifier upper cutoff frequencies.

The small signal equivalent circuit in high frequency range is

The equations in Figure 2 contain the variables Cin(M)= CM1 and Cout(M)= CM2. These variables represent the Miller capacitance values for the circuit. Miller’s theorem states that a feedback capacitor (such as Cbc= CF) can be represented as separate input and output capacitance values (which simplifies the analysis of the circuit). The values of the Miller capacitances are found using the relationships shown in the figure.

Using Miller effect, modify the equivalent circuit.

CF∥Cu = CF+ Cu ≈ CF    ←    CF ≫ Cu

vBvA= Avi ≫1

Upper 3dB frequency due to CM1 is

f2B= 12πrπ∥ RinCM1              Rin= R1∥R2∥RS

CM1= Cπ+ 1+AviCF ≈ AviCF   where  Avi≫1 and CF≫Cπ

Upper 3dB frequency due to CM2 is

f2C= 12πRC∥RLCM2              CM2 ≅ CF

Generally the upper half power frequency is determined by CM1 because the value of CM1 is much larger than that of CM2.

Therefore,

CM1= 12πrπ∥ RinfC2   where  fC2  is upper half power frequency

CF= CM1Avi